Smart dustbin with segregation of wet and dry

Pathi nandini
Department of ECE
QIS College of Engineering and technology 
Ongole,India
24491A04M0@qiscet.edu.in

Shaik mujwin
Department of ECE
QIS College of Engineering and Technology ,Ongole,India
24491A04N2@qiscet.edu.in

Kandamalla parvathi
Department of ECE
QIS College of Engineering and Technology 
Ongole,India
24491A04L6@qiscet.edu.in

Pidathala yaswanth
Department of ECE
QIS College of Engineering and Technology 
Ongole, India 
24491A04Q0@qiscet.edu.in

Kanubuddi anitha
Department of ECE
QIS College of Engineering and Technology
Ongole, India 
24491A04L8@qiscet.edu.in

Nalamala Narasimha
Department of ECE
QIS College of Engineering and Technology 
Ongole,India
24491A04J5@qiscet.edu.in

Abstract-- This paper presents a Smart Dustbin system that automatically segregates wet and dry waste using sensor technology. A moisture sensor detects waste type, and a microcontroller controls a servo mechanism for separation. The system also integrates Internet of Things technology for real time monitoring and efficient waste management.

Keywords— Smart Dustbin, Waste Segregation, Moisture Sensor, Arduino, Internet of Things.

# I Introduction

Rapid urbanization has increased municipal solid waste, creating environmental and health challenges. Proper segregation at the source is essential for efficient recycling and sustainable management. This paper proposes a Smart Dustbin system that automatically separates wet and dry waste using sensor-based technology to improve hygiene and operational efficiency.

# II Related work

## RFID-Based Toll system

An RFID based toll system is an automated toll collection mechanism that uses Radio Frequency Identification technology to identify vehicles without stopping them at toll plazas. Each vehicle is equipped with an RFID tag containing a unique identification number. When the vehicle passes through the toll gate, an RFID reader scans the tag and retrieves the stored information. The system automatically deducts the toll amount from the user’s linked account and records the transaction in a central database. This technology reduces traffic congestion, minimizes waiting time, and improves toll collection efficiency..

## ANPR-Based System

An ANPR based system, also known as Automatic Number Plate Recognition, is used to identify vehicles by capturing and processing their license plate information. The system uses a high-resolution camera to capture vehicle images, and image processing algorithms are applied to extract and recognize the number plate characters. The recognized data is then compared with a database for authentication or transaction purposes. ANPR systems are widely used in traffic monitoring, toll collection, and security applications, as they enable automatic vehicle identification without physical contact or manual intervention.

# III.    Proposed work and Methodology

## A.  Proposed work

## The proposed system presents a Smart Dustbin with automatic segregation of wet and dry waste integrated with RFID technology for user identification and monitoring. The system uses a moisture sensor to detect the type of waste and an ultrasonic sensor to sense object presence. An Arduino microcontroller processes the sensor inputs and controls a servo motor to direct waste into appropriate compartments. The RFID module records user details to promote responsible waste disposal. The collected data is stored and monitored through an IoT platform, enabling real time tracking and efficient waste management in smart city environments.

## .

## Methodology

- User Identification: The RFID tag is scanned to identify and authenticate the user before waste disposal.

- Waste Detection: An ultrasonic sensor detects the presence of waste placed inside the dustbin.

- Moisture Analysis: A moisture sensor measures the moisture content to determine whether the waste is wet or dry.

- Automatic Segregation: The microcontroller processes sensor data and activates a servo motor to direct waste into the appropriate compartment.

- Data Monitoring: The IoT module records disposal data and enables real time monitoring for efficient waste management.

  The smart dustbin system is designed using an Arduino UNO as the main controller to manage all operations.

  The ultrasonic sensor is used to detect the presence of waste when a user approaches the dustbin.

  Once waste is detected, the lid or flap mechanism is activated using a servo motor.

  The moisture sensor checks the waste type by measuring the moisture content.

  Based on the sensor value, the Arduino decides whether the waste is wet or dry.

  The servo motor rotates accordingly to direct waste into the correct compartment.

  The waste falls into either wet or dry section inside the dustbin.

  The system continuously monitors the bin level using ultrasonic sensor.

  When the dustbin reaches maximum level, the system can send notification (IoT optional).

-   The process repeats automatically for every waste detection

- . 1.Hardware Components

- Arduino

- 2.1: Arduino

- 1.It is used to control the servo motor based on the logic

- 2.Microcontroller that processes sensor data.

- Servo Motor

- 2.2:Servo Motor

- 1.Mechanically open and close the gate       or parking barrier.

- 2.ultrasonic sensor

Purpose: The ultrasonic sensor detects the presence of waste and measures the distance using sound waves.

Working: It sends ultrasonic pulses and calculates the time taken for the echo to return, which helps determine object detection and bin fill level.

- 2.3:IRSensor

## 1.Detects the distance of vehicle approaches from gate. And Sends the distance data to Arduino.

## Bread Board

## 2.4: Bread Board

## 1.For Connections only.

.  Moisture sensore

2.5:LCD Display

Function: The moisture sensor measures the moisture content present in waste to identify whether it is wet or dry.

Working Principle: It operates based on electrical conductivity. Wet waste conducts more electricity, producing a higher output value compared to dry waste.

Application in System: The sensor sends data to the microcontroller, which processes the value and controls the servo motor for automatic segregation.

## C.  Algorithm

Start the system and initialize all sensors and microcontroller.

Scan RFID tag and verify user identity.

Detect waste presence using ultrasonic sensor.

Read moisture value from moisture sensor.

Compare moisture value with threshold:

- If value ≥ threshold → classify as wet waste.

- Else → classify as dry waste.

Rotate servo motor to direct waste into appropriate compartment.

Update data to IoT platform for monitoring.

Repeat process for next input.

Stop.

## D.  Block Di   2.6:Block diagram

- 2.7: Circuit diagram

- F.   Equations used in this Project

- Ultrasonic Distance Measurement:

D=V×T2

- Ultrasonic Practical Formula (in cm):

D=T×0.03432

- Ohm’s Law (Moisture Sensor Principle):

R=VI

- Moisture Threshold Condition:

M≥T→Wet
M<T→Dry

- Servo Motor Angle Control (PWM relation):

- θ∝PWM

- IV.         Prototype

## 2.8: Prototype

- The prototype of the Smart Dustbin is built using a dual compartment structure for wet and dry waste segregation.

- It integrates sensors and microcontroller-based automation to classify and separate waste efficiently.

- The model demonstrates real time waste segregation and IoT monitoring, validating the feasibility of the proposed system.

V.    Results and discussion

- The RFID module successfully authenticated users, ensuring controlled access to the smart dustbin system.

- The ultrasonic sensor accurately detected waste presence and monitored the bin fill level in real time.

- The moisture sensor effectively differentiated wet and dry waste, though proper threshold calibration was necessary for accurate classification.

- The Arduino UNO processed sensor inputs efficiently and controlled the system without noticeable delay.

- The servo motor responded quickly and directed waste into the correct compartment, preventing mixing.

- Minor variations in moisture readings were observed under excessive liquid conditions, but overall performance remained stable.

- The IoT module successfully transmitted waste data for monitoring, supporting better waste management planning.

- The prototype proved to be cost-effective, energy-efficient, and suitable for smart city and institutional applications.

- .

VI.    Conclusion and Future scope

## Conclusion

- The Smart Dustbin system successfully automates wet and dry waste segregation using sensor technology.

- RFID integration ensures user identification and promotes responsible waste disposal.

- The ultrasonic and moisture sensors provide reliable detection and classification of waste.

- The servo motor mechanism accurately directs waste into appropriate compartments.

- IoT integration enables real time monitoring and efficient waste management planning.

- The system is cost effective, energy efficient, and suitable for smart city and institutional applications.

## B.   Future scope

- The system can be enhanced using Artificial Intelligence for advanced waste classification beyond wet and dry categories.

- Integration of image processing techniques can improve accuracy in identifying different types of waste materials.

Solar power support can be added to make the system energy independent and eco friendly.

- GPS integration can help in tracking bin location for efficient waste collection scheduling.

- Mobile application development can enable users and authorities to monitor waste data easily.

- The system can be scaled for large scale implementation in smart cities and industrial

VII.  Acknowledgment

I express my sincere gratitude to QIS College of Engineering and Technology for providing academic and technical support throughout the project.

I am thankful to my project guide for their valuable guidance and continuous encouragement during the development of this work.

I extend my appreciation to the faculty members of the department for their assistance and insightful suggestions.

I also acknowledge my family and friends for their constant motivation and support in completing this project.

Finally, I am grateful to all those who directly or indirectly contributed to the successful completion of this work.

##### VIII.   References

Sharma, R., & Patel, S. (2020). Smart waste management system using IoT. International Journal of Engineering Research, 8(3), 45–50..

Kumar, A., & Singh, P. (2019). Automatic waste segregator using microcontroller. Proceedings of International Conference on Smart Technologies, 120–125.

Patil, S., Deshmukh, R., & Jain, M. (2021). IoT based waste monitoring and management system. International Journal of Computer Applications, 174(12), 10–15.

Gupta, N., & Verma, A. (2018). Smart dustbin for waste segregation using sensors. International Journal of Engineering and Advanced Technology, 7(4), 89–93.

Singh, R. (2022). Automated waste segregation using sensor technology. Journal of Environmental Engineering and Technology, 5(2), 33–38.

World Health Organization. (2018). Waste and human health: Evidence and needs. World Health Organization.

Rao, P., & Mehta, S. (2020). Internet of Things based smart city waste management. International Journal of Smart Systems and Technologies, 6(1), 21–22

Computer Science and Information Technology & Security (IJCSITS) 6.1 (2016): 582-585.

10.Sumathi, S. M., K. M. Nikhitha, R. Manasa, A. Jithesh, and D. H. Megha. "Automatic Toll collection system using RFID." Int J Eng Res Technol 6, no. 15 (2018): 1-7.

- IEEE conference plates contain guidance text for composing and formatting conferencepapers. Please ensure that all template text is removed from your conference paper prior to submission to the conference. Failure to remove template text from your paper may result in your paper not being published.